PRABHAKARAN. R, B.E.,  							Cell No.9094270641

Proprietor

MAUNA CONSTRUCTIONS

Reg.No.TN-24-0063689

No.3/178, Thandarai Main Road, Pidarithangal, Chennai-600 056

RATE 2400/SQ.FT

| S.No. | DESCRIPTION OF MATERIAL | BRAND TO BE USED |
| --- | --- | --- |
| 1. | CEMENT | CORAMANDEL/DALMIA/ULTRATECH |
| 2. | STEEL | ARS 550TMT BARS |
| 3. | BRICKS | BOX BRICKS |
| 4. | SAND | M SAND / P SAND |
| 5. | TILING WORKS | 4’ x 2’ KAG vitrified tiles (Maximum Range – Rs.70) |
| 6. | SANITARY WARE FITTINGS | ( JAQUAR ) ESSCO / Parryware |
| 7. | CP FITTINGS | (JAGUAR )ESSCO/ Parryware |
| 8. | ELECTRICAL SWITCHES | ORBIT/LEGRAND |
| 9. | WIRES | ORBIT/FINOLEX |
| 10. | DB/SOCKETS | ORBIT/L&T |
| 11. | WATER SUPPLY PIPES | FINOLEX /ASHIRWAD |
| 12. | PAINTS | INNER WALL – ASIAN PREMIUM OUTER WALLS - ASIAN WEATHER PROOF EMULSION (APEX) |

DETAILED SPECIFICATIONS:

STRUCTURE:

*	R.C.C. Framed Structure as per architectural and structural drawings.

*	Ground Floor finishing level share be 3' feet from the existing ground level.

*	Partition walls will be 4½ x 9’ thick walls as per architectural drawings.

*	Parapet wall of 4½ thick and 3 feet height.

WALL FINISHES:

*	Internal Walls will be finished with smooth cement plaster, putty and Premium Emulsion paint.

*	Exterior walls will be finished with neat cement plaster & white cement, one coat primer and Exterior APEX.

*	Toilet walls will be finished with Ceremic Tiles of height up to 7’0”

*	Kitchen platform Walls will be finished with Glazed Tiles & height 4’0” above The cooking platform.

*	Ceiling surfaces will be finished with near cement plaster putty and Tractor emulsion.

FLOORING:

*	Living, Dining, Kitchen and Bed Room Area will be finished with 48'x24” joint free Vitrified tiles (Rs.70/- per Sq.ft)

*	Toilet area will be finished with glazed Tiles (Rs.40/- per sq.ft.,)

*         main door covered tiles - 40₹/sqft

JOINERIES

*	Entrance Main Door (12000)₹ will be provided with 1st quality teak wood Frames (size of Wood 3” x 4”) Teak Flush Doors with required Fittings & Locks.

*	All Bedroom Doors ( 4000₹)will be provided with 1st quality teak Wood Frames (size of Wood 4” x 2½”) & Solid flush Doors.

*	All Toilet Doors will be of PVC

*	MAIN DOOR - 12000₹

FRAME- 5X3

KITCHEN:

*	Counter Top Granite platform will be finished with 20mm thick Granite Slab 2’0” (Range 120 Sq.ft.,) wide at a height of 2’6” from the finished floor level along with Stainless Steel bowl sink (Max range 5000) & CP long body tap and RO Paint will be provided.

* 	Provision for Exhaust fan, Mixer, Grinder & will be provided.

*	Single Loft & Shelf can be provided under customization.

*	Two light points, two fan point, three 5 Amps, three 15 Amps.

BED ROOMS:

*	Cuddapah stone Wardrobe & a loft shall be provided under customization (Max size 4 x6) /One cupboard per room

*	Electrical point will be based on customer need.

LIVING/DINING

*	Two Fan points, Two Wall points, one Chandlier point, Three 5 Amps point, and one call bell

* 	Provision for TV unit will be provided.

BATHROOM:

*	Floor mounted White EWC or IWC in all Toilet and a white wash basin of above mentioned brands will provided for all toilets (Max Range Rs.8500-)

*	Hot & Cold Water mixer unit of one number will be provided.

*         water tank - ASHIRWAD (1500 Ltrs)

PAINTING:

*	Ceiling one coat primer,  two coat Premium Emulsion ASIAN.(white)

*	All Internal wall two coat putty, one coat primer and two coat Premium emulsion ASIAN.

*	Bedrooms frames, bathroom frame & doors, all windows & Ventilator’s two coat chalk putty, one coat primer & two coat enamel painting.

*	Main Door frames & doors varnish finish

*	Outer walls one coat primer, two coat exterior APEX Asian.

GRILL WORK:

*	Grill for windows & Ventilators will be of 12mm MS Square rod.

*         staircase handrails of SS Jindal 650, 650₹/ft.

*         Staircase granite price range - 100₹

*         window UPVC Brand - VENESTA

ELECTRICAL:

*	Team power supply & Concealed insulted Multistrand copper wires will be provided in all units with individual Distribution box having MCB’s.

*	All internal conducts in PVC pipes.

*	Good quality wiring will be used for entire building.

*	Modular switches will be provided of white color.

*	Television points in living and master Bed-Room.

IMPORTANT NOTES:

1.	Landscaped Gardens, borewell, Lift, weathering coarse are not included in the Quote, this will be charged extra if required by the Client, Water, Shed and electricity should be provided by client for construction.

2.	In case any modification are required in the given Specifications, the same shall reflect on the Quoted Rates, as per the client’s Needs & Requirements, Rates may go hit higher or lower depending on the changes.

3.	Terms of Payments can be discussed on Mutual concern, as per the convenience of the Client & the Company. The rate is valid only for 30 days Work will commence only after receiving first payment.

4.	Owner to be obtain the necessary planning from the Chennai Corporation. The necessary charges for payment CMWSSB Board Corporation of Chennai Corporation AE, MC, and other political parties shall be paid by the owner.

5.	Area is measured outer to Outer including roof slab projection

6.	EB Main Board Fittings & Labour should be paid by client.

7.	Weathering Coarse (Cool Roof Tile @ 120sq.ft.,)

Main gate - MS, Safety Gate – MS